# Large-scale flood mapping – Tutorial

June 2025

Guénolé Choné (g.chone@yahoo.ca)mailto:

Julia Mazgareanu (julie.mazga@gmail.com)

Concordia River Management Lab

# Purpose

The purpose of this document is to deliver step-by-step instructions about how to apply the large-scale flood modelling methodology developed by the River Management Lab at Concordia University.

# Requirements

## Software requirements and installation

- ArcGIS PRO, with Advanced licence and Spatial analyst extension. The 3D Analyst extension can also be useful but is not required.

- LISFLOOD-FP version 7 or above.   (See http://www.bristol.ac.uk/geography/research/hydrology/models/lisflood/, https://zenodo.org/record/4073011#.ZCbhlXbMKUl). Make sure LISFLOOD-FP is working. You should have at least basic knowledge of LISFLOOD-FP inputs and outputs (using the coupled 1D/2D sub-grid channel model).

- ArcGIS tools developed by the lab, available at https://github.com/gchone/Regional-Scale-Flood-Modelling. Download the last version of the tools.

- Lastools by rapidlasso Gmbh (https://rapidlasso.com/lastools).

## Data requirements

For a given watershed to be modelled, the following data are required:

- LiDAR LAS files (or LAZ files) with point timestamps (see section 5.2.2). Only LiDAR LAS file of the studied reaches is required.

- LiDAR 1m DEM of the rivers and their floodplains

- A coarser DEM covering the entire watershed (e.g. 10 m or 20 m resolution).

- (Optional) Vectorized stream network data, either from the National Hydro Network (https://open.canada.ca/data/en/dataset/a4b190fe-e090-4e6d-881e-b87956c07977) or from a provincial data source

- 2015 Land Cover of Canada : https://open.canada.ca/data/en/dataset/4e615eae-b90c-420b-adee-2ca35896caf6/resource/e8da7a31-e809-4fc8-91a8-3edf28c9f10c, or other land cover data.

- Discharge data from gauging stations in, or nearby, the study area, or from a hydrological model. These discharges should cover the time the LiDAR data were acquired. In addition, flood discharges (e.g. 100 yr flood) should also be available. Discharges should be available at least one point on the watershed, or nearby enough. These data could come from hydrological modelling or could be measured at discharge gauging stations. The Large-scale flood mapping process includes tools to assess discharges anywhere on the drainage network, using the discharges at specific points and the following equation:

Q= QstAAstβ

where Q and A are the discharge and drainage area at the computed point; Qst and Ast are the discharge and drainage area a gauging station; β can be assessed by statistical means prior to applying the Large-scale flood mapping process (especially for flood discharges), or set to 1 if this value is unknown.

- Estimated level of lakes and reservoirs to be used as downstream boundary conditions. The Large-scale flood mapping process provides flood elevation estimates for rivers, not lakes and reservoirs. The elevations of floods at lakes and reservoirs should be assessed independently.

# Must read: some important notes before beginning

- All files that go into the tools described herein have to have the same cell size, number of columns and rows, extent and spatial reference of the flow direction raster (or DEM), as well as be snapped to it. If this condition is not respected, the tools will not run, will display errors/warnings or will output inaccurate results. As such, make sure to specify this in the Environments (ArcMap: these can be found in the interface of each tool; ArcPro: specified in the Analysis tab -> Geoprocessing menu -> Environments).

- For the sake of increased clarity, ArcGIS geoprocessing tools will be signified by an orange colour (ex: Polygon To Line), while in-house tools, as well as associated scripts, will be demarcated by a blue colour (ex: D4 flow direction). The in-house tools used in this document can all be found in the toolbox LargeScaleFloodModelling_Toolbox, from the GitHub project page (https://github.com/gchone/Regional-Scale-Flood-Modelling). An additional toolbox, LargeScaleFloodModelling_Toolbox_DetailedTools, is also provided for custom usage (not covered in this documentation).

- The project workspace must be set to a folder (not a geodatabase), in Analysis tab -> Geoprocessing menu -> Environments -> Current workspace. ArcGIS File Geodatabases created during the application of this methodology (e.g. Geometry.gdb) must be created within this folder. In-house tools often have the input and output parameters automatically filled, expecting this specific workspace structure.

- When using in-house tools, select input data from the Catalog, not from the Table of contents.

# Process overview

The large-scale modelling approach takes place in three main steps.

First, the raw data must be processed in order to obtain width and bed elevation. These data are created in a vector format (points with width and bed elevation measurements along the river network).

Second, in the case hydraulic simulations are done in LISFLOOD-FP, data (including the width and bed elevation) are converted into the right format for LISFLOOD-FP. These inputs are mainly 6 rasters containing the following information:

- DEM

- Channel width

- Channel mask

- Bed elevation

- Flood discharge

- Manning’s n values for the floodplain

Third, the LISFLOOD-FP simulations can take place.

# Data preparation

## Identification of the river networks

The very first step of the methodology is to identify the river networks for which the flood assessment is needed. If a river network is not already targeted, it can be identified using a threshold on drainage area (e.g.: all streams with a drainage area greater than 200 km2).

In order to compute the drainage area, a first 10m resolution DEM at the watershed scale must be computed (section 5.1.1). Note that this DEM is a temporary one, used only for preliminary drainage area computation. It should not be used further in the process.

- The use of the three ArcGIS tools Fill, Flow direction and Flow accumulation gives a flow accumulation raster indicating the drainage area in a number of pixels (e.g.: with a 10m raster for example, the flow accumulation raster can be transformed into square-kilometers by dividing it by 10 000).

- using the Flow Accumulation result, Reclassify can be used to set a specific value (i.e. 1) to rivers with more than a predetermined threshold of drainage area.

- Raster to Polyline can turn the result into a shapefile, in order to have a representation of the river network as lines.

A temporary version of the river network with rivers represented as polygons is required to extract the LiDAR data needed for the estimation of bathymetry. This polygon river network can be created using the polygons from the NHN, with manual editing to cut out upstream parts and unwanted tributaries. For streams for which a NHN polygon doesn’t exist, but lines do exist in the NHN, a simple buffer (Buffer tool) with a crude approximation of the half-width value can be done and merged to the previous results.

## LiDAR DEMs

### Type of LiDAR DEMs

Two different sources of LiDAR data are used within the Large-scale flood modelling methodology:

- The 1m LiDAR DEM

Such a DEM can usually be downloaded from government agencies if LiDAR is available for a given area. These data are used for floodplain elevation in the Large-scale modelling process. More specifically, this DEM is used (with a coarser resolution) as the terrain for hydraulic simulations, and for computing flow depth in the post-processing final steps of the methodology. This 1m LiDAR DEM must therefore cover the floodplain of the rivers.

- The 1m DEMs by days of LiDAR acquisition

These DEMs must be computed from the LAS files. Rivers width and water surface elevation are extracted from these DEMs in order to compute the bathymetry. Consequently, the required LAS file needs to cover only the river channels.

For the majority of watersheds, several LiDAR surveys are available. Furthermore, the LiDAR data for any single LiDAR project also overlaps between two acquisition days. It is preferable to have all available LAS files when several LiDAR surveys overlap, unless there are reasons to believe one LiDAR surveys is preferable (e.g. the other survey being way older and/or of less points density).

Example: the Bécancour watershed (Quebec, Canada) is covered by 4 different LiDAR projects (thick lines). Each LiDAR project required several days of LiDAR acquisition (thin lines).

### Creation of LAS files by days of LiDAR acquisition

The points in the LAS files must be filtered by days of LiDAR acquisition, then a DEM must be generated for each day. These operations can be performed with any LAS file processing software. The following detailed steps explained how to do this using LAStools from rapidlasso GmbH (https://rapidlasso.com/lastools/) for LAS files processing and ArcGIS PRO for DEM creation. The LAS files must have time recorded as Adjusted Standard GPS time (see Las format specification, https://www.asprs.org/wp-content/uploads/2019/03/LAS_1_4_r14.pdf). This can be checked by running lasinfo from Lastools on a single LAS file to have a look at the time format.

Create by-day LAS files is the tool to filter the LAS files

Input:

Lastools bin folder: path of the folder “bin” from Lastools

Folder with LAS files: the folder with the LAS (or LAZ) files

UTC zone: The UTC local time (eg: -4 for Montreal during summer hours)

Output:

Intermediary results folder - Original LAS files split by days

Final result folder - LAS files by days

This tool can take quite some time if the river network cover hundreds of km of rivers (hours). Progress can be checked by looking at the files created in the intermediary results folder. It’s recommended to test the tool with a small subset of LAS files to begin with, to make sure the outputted dates make senses.

In the final result folder of the tools, LAS files by days of LiDAR acquisition are created along with shapefiles of the footprints. Aberrant points are occasionally included in the footprints and must be manually removed.

### Conversion of the LAS files into DEMs by days of LiDAR acquisition

ArcGIS offers tools to convert LAS files into DEMs. LAS Dataset to Raster can be used for a straightforward conversion. Start by converting one (and only one) LAS file with this tool, choosing the appropriate coordinate systems and imposing a Cell Size = 1 (for creating a 1m resolution DEM). In the tool parameters, choose triangulation with linear interpolation. The remaining DEMS should be then created with the same tool and parameters with in addition the “Snap Raster” option using this first DEM (if an important number of days of LiDAR acquisition are present, such a task can be automated with ArcGIS Model Builder).

DEMs should then be clipped with the Clip Raster tool, using their footprint as the Output Extent (and the option Use Input Features for Clipping Geometry checked).

### LiDAR survey selection

It is very common that there are overlaps between two or more acquisition days. Different LiDAR data differ in quality (density of LAS points) and in the discharge present during that day. These two factors greatly impact the water surface elevation and bathymetry estimation. It is therefore necessary to process the overlaps between different days so that the best available LiDAR surface is preserved (this is done by looking at the hillshades).

The main goal of this process is to clip the polygonal footprints of each LiDAR day so that there are no overlaps anymore. Using the clipped polygons, the DEMs themselves are to be clipped and combined into one single DEM, to be used for further processing (sections 5.6 and 5.7).

To do this, here are some general guidelines:

- the newest LiDAR data tend to be of better quality

- if the two overlapping surfaces are very similar, choose the one with the lowest discharge

- for shorter overlaps, it is best to clip in the middle of the overlap, since DEM margins tend to have a higher number of errors (this can be easily seen on hillshades, as illustrated in the image below. Left: low quality DEM margin; right: better quality surface for comparison)

- for longer overlaps, clip so as to include the best quality LiDAR for as long as possible

- if possible, avoid clipping the polygon longitudinally along the river, like so:

- avoid clipping the polygonal footprints in the vicinity of bank-to-bank interpolation errors (these look like TINs on the surface of the water). If you have little choice, place a bridge polygon (see section 5.4.1) that covers the TIN. Close to the edges of footprints, the water surface correction algorithm may not correct bank-to-bank interpolations, so in certain cases, it is better to ensure that this happens manually, by placing bridge polygons.

- avoid clipping the polygonal footprints at major breaks of water surface (at chutes or major steps). Ideally the transition between two days should be in a section of uniform slope.

### Troubleshooting unusual issues in the LiDAR data

Possible types of issues with resulting DEMs:

- Data gap. Solution: Additional LAS files are needed.

- Gap between two LiDAR dates. Solution: fill the gap by using Nibble. Make sure the produced water surface is realistic (compared to elevation upstream and downstream the gap).

- Small tributary without a visible channel. Solution: such a case cannot be modelled. This needs to be removed from the network.

- Blurry data. Cases:

- Data is blurry both on the governmental 1m DEM. Solution: continue with current (blurry) data.

- Data is of good quality on the governmental 1m DEM, so there must be a superposition of multiple days. Solution: replace surface with good quality date.

When editing the individual footprint polygons is done:

- Merge all LiDAR footprints into one single file “Geometry.gdb\DEM_footprints”. Add a new field (Type = Text) called “ID_DEM” to store information that links each footprint with the corresponding DEM.

- Find Overlaps (ArcGIS Pro) to identify overlaps in the merged DEM footprints feature class

Input: Geometry.gdb\DEM_footprints

Output: Geometry.gdb\DEM_footprints_overlaps

Geometry.gdb\DEM_footprints_centroid

- Edit DEM_footprints to adjust the borders between two footprints so that they don’t overlap (you can do this by selecting both overlapping footprints and drawing a line between them via “Reshape”). They need to share the exact same border.

## Identification of lakes and reservoirs

Because the hydraulic modelling uses a steady flow (constant discharge), lakes and reservoirs must be excluded from the hydraulic modelling, and require thus proper identification.

- Lakes and reservoirs can be first extracted from the NHN. The ones on the studied area can be identified by a Selection by Location, using the intersection with the polygonal river network.

Output: Geometry.gdb\lakesNHN_onnetwork

- Small lakes relative to the river width can be included in the hydraulic simulation, as their effect on hydraulic can be neglected: a threshold of 5 ha for small rivers, or 10 ha for wider ones (200m wide or more), can be used.

- Governmental data on dams can be helpful in finding small dams without reservoirs identified in the NHN. Reservoirs created by these dams must be digitized manually. To identify the reservoir, a longitudinal profile of the water surface elevation over roughly the length of the reservoir can be drawn with the 3D Analyst toolbar. Knickpoints in the longitudinal represent the extremities of the reservoir. These reservoirs can be then merged with lakesNHN_onnetwork. Output: Geometry.gdb/lakesfinal

## River polygons

A polygon of the wetted channel from the LiDAR data is required. The produced polygon is mainly used to measure the channel width.

As flood modeling is not particularly sensitive to errors in channel width measurements, a coarse polygon representing the river can be used. Where NHN polygons are available, these polygons can be used as a first draft of the channel. They cannot however be used blindly, and must always be manually checked and corrected to better fit the wetted channel on the LiDAR data. Where the NHN polygons are not available, a manual digitization from scratch must be done.

The river polygon can be based on hillshades of the by-days DEMs processed in 5.2.3. The digitization must reflect the surface of the DEM/hillshade and only include what is submerged by water (e.g. sediment bars that have higher surface elevations than the wetted channel elevation must be excluded).

While you are editing the channel polygon, it is very useful to also identify the bridges that later need to be corrected in the water surface estimation step (section 5.7).

Create polygons of the bridges (having the same coordinate system as the DEM). The bridges should be small rectangles that contain at least one full cell of a lower elevation around the bridge (corresponding to channel below). The idea is to assign that polygon the lowest elevation value found within it, so this should ideally correspond to the channel. The bridge polygon doesn’t need to be wider than the channel. When the channel polygon is already at hand, the bridges can also be traced along the channel polygon edge.

Output: Geometry.gdb\bridges

Digitization of the streams included in the network of interest can be done using the hillshade rasters of the by-days DEMs (section 5.2.3). Ensure that the channel polygon corresponds to the clipped polygonal footprints of the DEMs.

- At the downstream end of the river, the river polygon must extend at least one full cell outside the DEM.

- As much as possible, try to end the channel polygons in a line that is transversal to the flow

- All sections of rivers with dead flow (e.g. oxbows) must be cut out

- Lake polygons (file lakesfinal, see 5.3) can be added directly to the results. No manual editing is required as the polygons around lakes don’t need to be as precise as for the rivers.

- The river network, including lakes, must, ultimately, be one and only one polygon. This can be done when you are done with editing the polygon by using Dissolve (with create multipart features unchecked): the resulting attribute table must contain one and only one row.

Output: Geometry.gdb\channelpoly

Tip: Change DEM symbology (colour ramp) and select DRA (dynamic range adjustment) from raster layer tab for the DEM to change the stretch according to the zoom applied. Try using different stretch types such as minimum-maximum and histogram equalize for the best visualization of the DEM.

To correct any errors while drawing the channel polygon, use the reshape tool and make sure that the snap to grid is ON. The trace tool can also be helpful if you would like to connect to two polygons to make the sure that the polygon has the same boundary. Use the split tool if you want to remove mid-channel bars (the endpoint lines need to intersect).

## Linear river network

This step aims to produce a linear network of the rivers that matches the channel polygon (section 5.4).

- Over wide rivers, the linear network of the NHN can be used. It is however necessary to check that the river lines always fall inside the river polygons, and correct them if necessary.

- The NHN data is often too approximate for small streams. Manual digitization of the stream centerlines can be done.

- Alternatively, a centerline can be automatically derived from the river polygon (containing the lakes too) so that the line always falls inside the polygon:

(ArcGIS PRO only, for those who have the Production Mapping Licence) Polygon To Centerline to create a centerline based on the channel polygon

Input: Geometry.gdb/channelpoly

Output: Geometry.gdb/centerline

For those working with ArcMap or ArcGIS PRO without access to the Polygon To Centerline tool, this can be achieved by first converting the channel polygon to a polyline and then collapsing this to a centerline:

- Polygon To Line to create lines of the banks

Input: Geometry.gdb/channelpoly

Output: Geometry.gdb/polybanks

  - Collapse Dual Lines To Centerline to create a centerline

Input: Geometry.gdb/polybanks

Parameter: Maximum Width = the maximum width of the channels, not accounting for the lakes.

Output: Geometry.gdb/centerline

NB: For the Blanche, the raw centerline output without any edits is named “collapsedbanks”.

  - Edit the result to improve it (e.g. delete stray lines, connect lines, add lines in multichannels). Note that secondary channels are only needed to assess the width using the recommended cross-section method (see section 5.9). If any other method, including the Thiessen polygon method described in 5.9.2, is used to assess the river width, then digitizing the secondary channels in not necessary.

      - Delete stray lines and/or lines that go through oxbows or zones with no flow

      - Connect lines if disconnected

      - Keep and/or add important secondary channels. Add secondary channels only when they have a significant width and length compared to the main channel. NB: If the river branches out transversally to the flow direction, secondary channels do not have to be added.

Main channel (in blue) and secondary channels (in pink). Narrow secondary channels, short ones created by small islands, narrow ones, and the ones more with a more transversal than longitudinal orientation are not added.

      - Do not create secondary channels at confluences (figure below)

Incorrect secondary channels (in blue) connecting different reaches at a confluence.

  - Dissolve with Create multipart features unchecked. Verify that the line only splits at confluences (i.e. all intersections between lines).

Output: Geometry.gdb\linear_net_d_tmp

    - Add a field (Type = Short) called “Main” to the centerline. Main channels should receive a value of 1, whereas secondary channels should receive a value of 0.

    - Add a field (Type = Short) called “DownEnd” to the linear_net_d_tmp. The most downstream line should receive a value of 1, and the rest a value of 0.

    - Add a field (Type = Long) called “RID” to the linear_net_d_tmp. Copy the FID or OBJECTID values inside via Field Calculator.

Tip: Sometimes, it helps to check the shape_length in the attribute table of the result: by ordering the attributes by increasing length, small lines that extend beyond the network can be removed. These may appear due to manipulation errors, like the lines not being perfectly snapped to each other, and are most of the time so small that they are not visible by just going along the river network and observing it.

  - Smooth the network line (this is especially needed for a correct width assessment)

    - Divide network into 2 or 3 parts, according to the channel size (wide rivers need a lot of smoothing, whereas narrow rivers need little to no smoothing)

    - Smooth Line each divided segment (play around with different smoothing tolerance values, e.g. 100m for widths >100m, 10m elsewhere)

Output: linear_net_d_smXm, where X is the smoothing tolerance value

    - Check if smoothed lines go out of the channel polygon via Erase. Manually correct these situations or use less smoothing.

    - Merge smoothed divided segments into one.

Output: Geometry.gdb\linear_net_d

  - Within linear_net_d, select only the main channel lines (Main=1), Dissolve with Create multipart features unchecked.

Output: Geometry.gdb\linear_main_d

    - Add a field (Type = Short) called “DownEnd” to the linear_main_d. The most downstream line should receive a value of 1, and the rest a value of 0.

    - Add a field (Type = Long) called “RID” to the linear_main_d. Copy the FID or OBJECTID values inside via Field Calculator.

    - Check the output: lines should only split at confluences. If they don’t, there are probably very small lines that intersect at locations that are not confluences. These small lines need to be corrected on the linear_net_d (use snapping when correcting). Once that is done, rerun the Dissolve to create the linear_main_d and recheck if the lines split at confluences.

## Watershed-scale DEM preparation

This step aims to create a 10m DEM for the whole, to be used as input in Lisflood, as well as generate basic files used in several subsequent steps. Make sure you have DEMs:

  - At 1m resolution (from LiDAR data) for the study reaches and their floodplain

- At any resolution for the rest of the watershed if it’s not entirely covered by LiDAR data

- Batch Process Aggregate individual LiDAR tiles from 1m to 10m using the average (from  governmental DEMs, not the homemade by-days DEMs).

- Mosaic to New Raster to merge 10m DEM tiles into one single DEM

Inputs: all 10m LiDAR tiles

Parameters: Pixel Type: 32 float; Number of Bands: 1

Environments: once you have created the mosaicked governmental DEM, snap the by-days DEM to it

Outputs: lid10m_avg

- Prepare the streams network to burn into DEM (at the watershed scale):

  - In order to ensure that stream burning is applied correctly, first make sure that the river network/river polygon used in this step extend for at least one cell beyond the extent of the DEM is used in the stream burning.

  - Create a watershed-scale river network. This is only used in the stream burning process (a more comprehensive watershed-scale river network contributes to having more precise flow accumulation estimates).

    - Take the linear river network from the NHN database for the studied watershed and the neighboring watersheds (this is needed in order to prevent flow capturing from other watersheds in very flat terrain). Merged files for different watersheds if necessary and save a local copy.

    - Double-check that the network is completely contained within the extent of your DEM, e.g. in the image on the right, lines that lie outside the DEM extent (red circle) must be cut. In other words, the only place where the line can extend outside of the DEM is at the watershed outlet.

    - Select Layer By Attribute to delete Priority Level = 2 lines

    - Erase NHN network lines within the channel polygon:

Input: NHN

Geometry.gdb\channelpoly

Output: Geometry.gdb\NHN_erased

    - Merge NHN_erased with linear_main_d. Output: Geometry.gdb\watershed_net

- Watershed-scale DEM processing

This tool includes several important steps for the creation of the river networks used latter on in the process:

  - Stream-burning of the DEM

  - Common hydrological processing of the burned DEM (flow direction, flow accumulation)

  - Creation of the river networks themselves (modelled river networks, with linear-referencing and coded information about upstream/downstream reaches), with three versions:

    - routes, a network with secondary channels, beside islands for example);

    - routes_main, a network without secondary channels, creating a topological tree;

    - routesD8, a network that follows the flow direction raster.

Note that the tool requires that the geodatabase Mask.gdb already exists in the project folder. The following greyed-out steps describe the internal process of the Watershed-scale DEM processing. This detailed procedure can be followed, and modified as needed, by advanced users in place of using the Watershed-scale DEM processing tool. Inputs and outputs of the Watershed-scale DEM processing tool are automatically filled.

Watershed-scale DEM processing internal process:

- Polygon to Raster to transform the channelpoly into a raster (match Cell size, Output Coordinates, Processing Extent and Snap to compute the 10m watershed-scale DEM, lidar10m_avg)

Input: channelpoly

Output: Mask.gdb\frompoly

- Polyline to Raster (match Cell size, Output Coordinates, Processing Extent and Snap to compute the 10m watershed-scale DEM, lidar10m_avg)

Input: Geometry.gdb\watershed_net

Output: 10mDEMs.gdb\net_lines

- Raster calculator to burn the streams into the DEM. This is done via a 2 steps approach: 1) the channel raster is burned 100m, 2) the network lines are burned another 100m from the raster obtained in 1)

Input: Con(IsNull("frompoly"),  "lidar10m_avg", "lidar10m_avg" - 100)

NB: lidar10m_avg must be replaced by the watershed-scale DEM if they differ.

Output: 10mDEMs.gdb\lidar10mbpoly

Input: Con(IsNull("net_lines"), "lidar10mbpoly", "lidar10m_avg" - 200)

Output: 10mDEMs.gdb\lidar10m_burn

- Fill

Input: 10mDEMs.gdb\lidar10m_burn

Output: 10mDEMs.gdb\lidar10m_fill

- Flow direction

Input: 10mDEMs.gdb\lidar10m_fill

Output: 10mDEMs.gdb\lidar10m_fd

- Flow accumulation

Input: 10mDEMs.gdb\lidar10m_fd

Set Output Data Type to Integer

Output: 10mDEMs.gdb\lidar10m_facc

- Create network from feature class to create a river network data structure from a line feature class, defined by a link table and a RouteID. This needs to be done twice, once for the network containing secondary branches and once for the main network:

Input: linear_net_d

RID (RouteID field)

DownEnd (Field identifying most downstream reach)

Main (Field identifying the main or secondary channel)

Output: Geometry.gdb\routes

Geometry.gdb\routes_links

Input: linear_main_d

RID

DownEnd

Output: Geometry.gdb\routes_main

Geometry.gdb\routes_main_links

- Flow Direction Network to extract a route following the D8 flow direction and relate the main route with this resulting route. Check that the output relate table contains both and “RID” and a “RID_1” field.

Input: routes_main

routes_main_links

RID

lidar10m_fd

Output: Geometry.gdb\routesD8

Geometry.gdb\linksD8

Geometry.gdb\pathpointsD8

Geometry.gdb\fd_net_relatetable

- Order Reaches to order the reaches in the main route. After running this tool, check that routes_main contains a field called “Qorder” with a distinct number for each attribute.

Input: routes_main

routes_main_links

RID

lidar10m_facc

routesD8

linksD8

pathpointsD8

fd_net_relatetable

Output: Qorder

- (optional) Clip Raster the flow direction and the flow accumulation results to the studied area (use lidar10m_avg for the Output Extent) in case the studied area is a small subsection of the entire watershed

NB: This step was not done on the Blanche River.

## Water surface detection

The aim of this step is to extract water surface elevation along the studied rivers. This is done using the DEMs from section 5.2.1. This process gives better results with a raster at fine resolution. A 3m resolution is often a good compromise (finer resolution can be too long to process and produce too big files). For huge watersheds, a 5m resolution could be needed.

- Batch Process Aggregate individual by days LiDAR tiles from 1m to 3m using the minimum.

- Clip the by days DEMs according to the edited (non-overlapping) footprint polygons created in Section 5.2.1. Check “Use Input Features for Clipping Geometry” to use the exact shape that you have edited.

- Create a new geodatabase called “WaterSurface.gdb” to store all files created in this section

- Mosaic to New Raster to merge clipped 3m DEM tiles into one single DEM

Inputs: all 3m LiDAR tiles (by days DEMs)

Parameters: Pixel Type: 32 float; Number of Bands: 1

Outputs: WaterSurface.gdb\lidar3m_min

- Bridges and culverts correction to correct bridges and culverts on the 3m DEM

Input: WaterSurface.gdb\lidar3m_min

Geometry.gdb\bridges

Output: WaterSurface.gdb\lidar3m_forws

Burn the lidar3m_forws in lakes with the routes main:

This step is optional, and is necessary only if there are lakes with 3 tributaries or more within the studied network. The goal is to make sure that the confluences are placed in the same order than in the routes_main shapefile, so that the correspondence between the reaches of the two networks (the routes_main and the one produced in the next steps from the 3m resolution DEM) can be done automatically.

- Clip routes_main over lakesfinal

Input: Geometry.gdb\routes_main

Geometry.gdb\lakesfinal

Output : Geometry.gdb\routes_main_lakes

- Edit manually the routes_main_lakes at the upstream and downstream ends of the lake to be sure that they are extending out of the lake by a couple of cells (2 and more) and into the channelpoly along the Routes_main.

- Add field to routes_main_lakes, Name: rastervalue, Type : Short

- Calculate field in the routes_main_lakes so the rastervalue of all attributes is 1

- Polyline to Raster (match Cell size, Output Coordinates, Processing Extent and Snap to compute the lidar3m_forws)

Input: Geometry.gdb\routes_main_lakes (Value field = rastervalue)

Output: Watersurface.gdb\Lakes_burn

- Raster calculator to burn the Lakes_burn into the lidar3m_forws (or lidar3m_forws_check, if applicable).

Input: Con(IsNull("Lakes_burn"),  "lidar3m_forws", " lidar3m_forws " - 10)

Output: Watersurface.gdb\ lidar3m_forws_lakes

Use the lidar3m_forws_lakes instead of the lidar3m_forws in the next steps

- Flow Direction for Water Surface Assessment ()

Input: Geometry.gdb\routes_main

WaterSurface.gdb\lidar3m_forws_lakes

WaterSurface.gdb\DEM_footprints

Output: WaterSurface.gdb (this is a workspace where the outputs are stored, i.e. a flow direction raster for each footprint)

- Mosaic to New Raster to merge all flow direction rasters into one

- NB: If there are errors with certain areas, the tool can be rerun by selecting the affected footprints within the DEM_footprints feature class

- Create from points and split points to create from points and splits along the network

Input: Geometry.gdb\routes_main

Geometry.gdb\routes_main_links

RID

Output: Geometry.gdb\from_pts

Geometry.gdb\splits (with the routes_main, there should be no splits, so this output should come out empty)

- Create network from flow direction raster

Input: WaterSurface.gdb\lidar3m_fd

Geometry.gdb\from_pts

Output: WaterSurface.gdb\wsroutesD8

WaterSurface.gdb\wslinksD8

RID

WaterSurface.gdb\ws_pathpointsD8

NB: If there are issues with the lidar3m_fd, the tool will return an error message containing point coordinates between two footprints. In that case, the Flow Direction for Water Surface Assessment needs to be rerun by selecting from the DEM_footprints only the footprint(s) downstream of the indicated point(s).

NB #2: if still returning warning message (warning not error message) and that it’s giving point coordinates between two footprints. In that case, it means the footprint is crossing 2 times the main_routes in a single 3m cell, that’s the source of the problem. To bypass the problem, copy the footprint feature class, manually edit the copied footprint so it's not crossing 2 times the main_routes (see below). Then restart the steps with the copied footprint from the “Flow Direction for Water Surface Assessment”.

- Make sure the resulting lines reproduce the river network (if there is a hole in the data, the result can stop before reaching the downstream end.)

- Create points along route feature class to create the points used to extract the water surface, discharge and width for the computation of the bathymetry.

Input: Geometry.gdb\routes_main

Geometry.gdb\routes_main_links

RID

Interval: 5m

Output: Geometry.gdb\target_pts_raw

- Delete points outside of the DEM (3m one) in Geometry.gdb\target_pts_raw, if any, as well as points under the identified bridges

    - To display the target_pts_raw, run Make Route Event Layer

Inputs: Geometry.gdb\routes_main

RID

Geometry.gdb\target_pts_raw

RID

Point

MEAS

Output: target_pts_Events

    - Save the points by right-clicking target_pts_Events, select Data  Export Features

Output: Geometry.gdb\target_pts

    - Manually select and delete the points outside of the 3m DEM

    - Use Select By Location to select points under the polygons in Geometry.gdb\bridges and delete them

    - Save Edits

- Extract Water Surface

Input:

Geometry.gdb\routes_main

RID

Qorder

Geometry.gdb\routes_main_links

Geometry.gdb\from_pts

WaterSurface.gdb\wsroutesD8

RID

WaterSurface.gdb\wslinksD8

WaterSurface.gdb\ws_pathpointsD8

X

Y

WaterSurface.gdb\lidar3m_forws (DO NOT USE lidar3m_forws_lakes)

Geometry.gdb\DEM_footprints

ID_DEM

Geometry.gdb\target_pts

ObjectID_1

RID

MEAS

Output:

WaterSurface.gdb\net_relate_table

WaterSurface.gdb\smoothed_pts

## LiDAR discharge

Discharges for all the days of LiDAR acquisition must be available for at least one point on the watershed, or nearby enough. These data could come from hydrological modelling or could be measured at discharge gauging stations. The tool Spatialize discharges from gauging stations – Q LiDAR will compute discharges at the same place the water surface was previously computed (file WaterSurface.gdb\smoothed_pts), from the available data. This is done using the following equation:

Q= QstAAstβ

Where Q and A are the discharge and drainage area at the computed point; Qst and Ast are the discharge and drainage area a gauging station; β must be passed as a parameter to the tool (use 1 if this value is unknown).

The observed discharges (from the gauging stations) must be stored in a csv file, with the days of LiDAR acquisition as rows (ID_DEM) and gauging stations as columns. A shapefile of the gauging stations must also be available (with their names as a field).

Example of discharges file:

| ID_DEM | s01BO001 | s01BO002 | s01BO003 |
| --- | --- | --- | --- |
| d20150818 | 26.5 | 0.5 | 3.2 |
| d20150819 | 24.4 | 1 | 3.7 |
| d20150829 | 35.8 | 3.3 | 0.6 |
| d20150830 | 30.1 | 0.8 | 1 |

- Spatialize discharges from gauging stations – Q LiDAR

Inputs: 10mDEMs.gdb\lidar10m_facc

Geometry.gdb\routesD8

RID

Geometry.gdb\linksD8

Geometry.gdb\pathpointsD8

Discharge.gdb\QStations

OBJECTID

Field with the gauging station name

Field with the drainage area of the stations

Maximum distance between the gauging station points and the rivers (points located further than this distance will be ignored)

CSV file

Geometry.gdb\DEM_footprints

ID_DEM

Beta coefficient

fd_net_relatetable

Output: Discharge.gdb\Qpts_spatialized_D8

- Project point to the closest point on the network

Inputs: Qpts_spatialized_D8

computedQLiDAR

CLOSEST

Geometry.gdb\routes_main

RID

WaterSurface.gdb\smoothed_pts

RID

MEAS

Fields to match in the data points:

RID_routesmain

ID_DEM

Fields to match in the target points:

RID

ID_DEM

Output: Discharge.gdb\Qpts_spatialized

## Channel width

Width can be assessed by two different ways. Either cross-sections are drawn and their length measured (cross-sections method), or the rivers polygons are split in chunks using Thiessen polygons, and the width is assessed for each chunk by dividing their area by their length (Thiessen polygons method). The cross-sections methods is recommended in the vast majority of the cases. However, this method cannot provide accurate results in areas with complex multiple channels and the computed widths require manual correction in these cases. In the presence of multiple channels, the Thiessen polygons method is more practical. A river can be treated with both methods, in total or in parts, in order to keep the result of one method or the other depending on the context.

### Cross-section method

Width assessment by cross-sections. Cross-sections are not placed at confluences or in channels where no secondary channels lines were created. Some cross-sections are not correctly oriented (red circle) and width assessment should be manually corrected in such cases.

- Create a “Width.gdb” to store the results created in this section

- Calculate the width at transects:

  - Width by cross-sections

Input: Geometry.gdb\routes

RID

Geometry.gdb\channelpoly

1000

5

Output: Width.gdb\width_transects

Width.gdb\width_pts

  - Visually check the width_transects. Delete corresponding points (within width_pts) (or manually correct the associated width at these points) if some transects seem to not correctly represent the river width. This could be in particular the case if the routes extend beyond of the channelpoly. This can also happen if an island is present and a secondary channel was not added (figure below).

Example of points that need to be deleted.

- WidthPostProc

Inputs: Geometry.gdb\routes

RID

Main

Geometry.gdb\routes_main

RID

Shape_Length

Qorder

Geometry.gdb\routes_main_links

Width.gdb\width_pts

CSid

RID

Distance_m

Width_m

WaterSurface.gdb\smoothed_pts

ObjectID_1

MEAS

RID

Output: Width.gdb\width_postpro

Known bug: WidthPostProc can go into an infinite loop if several reaches in a row do not have associated width measurements points. To solve this problem, either simplify the routes network (removing secondary channels) or add points in the width_pts layer (don’t forget to fill appropriately the fields in the attribute table).

### Thiessen polygons method

Channel width is estimated by cutting the channel polygons and the channel lines into reaches (based on Thiessen polygons created from cross-section points at a specific interval) and transforming these into areas and lengths (width of reach polygon = area of reach polygon/ length of reach line).

- Create points along route feature class to place points along the network, at a specific interval. The distance used can vary with the size of the river. A distance of 100m can be used for small head streams, while a distance of 500m can be used for major rivers. If different distances are used within the same watershed, several files must be created and then merged manually into one single cross section shapefile (=xsect.shp).

Input:

Geometry.gdb\routes_main

Geometry.gdb\routes_main_links

RID

Interval between points = 200 (or 500)

Output: Width.gdb\width_pts_table

- Make Route Event Layer to visualize the points

Inputs: Geometry.gdb\routes_main

RID

Width.gdb\width_pts_table

RID

Point

MEAS

Output: width_pts_table Events

- Export width_pts_table Events into a feature class

Output: Width.gdb\width_pts

- Edit the points in order to add more points where the width of river changes rapidly (the idea is to capture, as much as possible, sudden variations in width). Points can be deleted as well.

- Create Thiessen Polygons

Input: Width.gdb\width_pts

Output: Width.gdb\thiessen

Output Fields: All fields

- Verify and edit the Thiessen polygons so that they split the river polygons in reaches. This can be done with the Reshape tool within an editing session. Check in particular confluences, sharp meanders, areas around lakes, and places of sharp changes of width. (The Thiessen polygon should extend completely from one bank to the opposite bank of the river reach described by the cross section point on which the Thiessen polygon is based.) See some examples below.

- Intersect Thiessen polygons with rivers polygons

Input:

Width.gdb\thiessen

Geometry.gdb\channelpoly

Output: Width.gdb\reaches_poly

- Intersect Thiessen polygons with rivers main lines

Input:

Width.gdb\thiessen

Geometry.gdb\routes_main

Output: Width.gdb\reaches_lines

- Dissolve the polygons according to Input_FID

Input: Width.gdb\reaches_poly

Output: Width.gdb\reaches_poly_d

- Dissolve the lines according to Input_FID

Input: Width.gdb\reaches_lines

Output: Width.gdb\reaches_lines_d

- Add Field to Width.gdb\width_pts to store Width (Type: Float)

- Join Field to add areas to Width.gdb\width_pts

Input:

Width.gdb\width_pts

ObjectID_1

Width.gdb\reaches_poly_d

Input_FID

Select transfer fields

Transfer Fields: Shape_Area

- Join Field to add lengths to Width.gdb\width_pts

Input:

Width.gdb\width_pts

ObjectID_1

Width.gdb\reaches_lines_d

Input_FID

Select transfer fields

Transfer Fields: Shape_Length

- Calculate Field to calculate "Width" field = Shape_Area/Shape_Length

- Delete points that fall into lakes (by selecting them by location). Check extreme values (by sorting the attribute table and Width.gdb\width_pts by Width, ascending and descending, and make sure no incoherent data was computed.

  - Interpolate Points

Inputs: Width.gdb\width_pts

ObjectID_1

RID

MEAS

Width

WaterSurface.gdb\smoothed_pts

ObjectID_1

RID

MEAS

Geometry.gdb\routes_main

RID

Qorder

Geometry.gdb\routes_main_links

Output: Width.gdb\width_postpro

## Bed elevation

The bed elevation is estimated through an inverse hydraulic problem given the already known variables of channel width and LiDAR discharge.

- Create a “Bathy.gdb” to store the results created in this section

- Join Width.gdb\width_postpro, Discharge.gdb\Qpts_spatialized and WaterSurface.gdb\smoothed_pts together by ObjectID_1 (NB: if doing this in a gdb doesn’t work, exporting the three tables to .dbf and doing the join like that works)

- Export result to “bathy_input_pts”

- Bed Assessment

Inputs: Geometry.gdb\routes_main

RID

Qorder

Geometry.gdb\routes_main_links

Bathy.gdb\bathy_input_pts

ObjectID_1

RID

MEAS

computedQLiDAR

Width_m

zws_smoothed

ID_DEM

0.03

Minimum energy slope: for the moment, use 0.00001

Output: bathy_pts

## Transformation to Lisflood format

- Create a new geodatabase “Lisflood_inputs.gdb” to store files created in this section, as well as a folder named Lisflood_inputs

- Create a local folder called “temp”. This folder will be used by the tool to store intermediary files. These files are normally automatically deleted after the tool completion but can persist if the tool fails.

D4 flow direction and Lisflood data conversion. This tool creates a D4 flow direction with an associated river network with points at each raster cell, and computes bed elevation and width values at these points. The following greyed-out steps describe the internal process of the D4 flow direction and Lisflood data conversion. This detailed procedure can be followed, and modified as needed, by advanced users in place of using the D4 flow direction and Lisflood data conversion tool. Inputs and outputs of the D4 flow direction and Lisflood data conversion tool are automatically filled.

D4 flow direction and Lisflood data conversion internal procedure:

- D4 flow direction to extract the D4 path of the flow direction (Lisflood requires a D4 path for its inputs)

Input: 10mDEMs.gdb\lidar10m_fd

10mDEMs.gdb \lidar10m_fill

Geometry.gdb\from_pts

Temp folder

Output: Lisflood_inputs.gdb\d4fd

- Flow Direction Network to extract a route following the D4 flow direction and relate the main route with this resulting route. Check that the output relate table contains both and “RID” and a “RID_1” field

Input: Geometry.gdb\routes_main

Geometry.gdb\routes_main_links

RID

Lisflood_inputs.gdb\d4fd

Output: routesD4

linksD4

pathpointsD4

D4fd_net_relatetable

- Add a field “Qorder” (Type = Short) to routesD4

- Join the D4fd_net_relatetable to the routesD4 (Input Join Field = RID, Join Table Field = RID_1), then join the routes_main to the routesD4 (Input Join Field = d4fd_net_relatetable.RID., Join Table Field = RID)

- Via Field Calculator, copy Qorder values of the route_main into the newly created Qorder field of the routesD4

- For the bed elevation:

  - Add Join between Bathy.gdb\bathy_pts (Input Join Field: RID) and the D4fd_net_relatetable (Join Table Field: RID)

  - Project point to the closest point on the network

Inputs: bathy_on_mainroute

z

MAX

routesD4

RID

pathpointsD4

RID

dist

RID_1

RID

Output: bathy_on_D4

  - Interpolate Points

Inputs: bathy_on_D4

id

RID

dist

z

pathpointsD4

id

RID

dist

routesD4

RID

Qorder

linksD4

  - Point to Raster (match Output Coordinates, Processing Extent and Snap to the lidar10m_fd)

Inputs: bathy_final

z

Cellsize: 10

Output: bathy_lisflood

  - Verify that output makes sense

- For the width:

  - Add Join between Width.gdb\width_postpro (Input Join Field: RID) and the D4fd_net_relatetable (Join Table Field: RID)

  - Project point to the closest point on the network

Inputs: width_on_mainroute

Width_m

MEAN

routesD4

RID

pathpointsD4

RID

dist

RID_1

RID

Output: width_on_D4

  - Interpolate Points

Inputs: width_on_D4

id

RID

dist

Width_m

pathpointsD4

id

RID

dist

routesD4

RID

Qorder

linksD4

Output: width_final

  - Point to Raster (match Output Coordinates, Processing Extent and Snap to the lidar10m_fd)

Inputs: width_final

Width_m

Cellsize: 10

Output: width_lisflood

- Verify that output makes sense

## Channel mask

The channel mask tells Lisflood what component is processed by the 1D model (=SuperGrid, i.e. channels larger than 15m width) and what component is processed by the 2D model (=SubGrid, i.e. channels smaller than 15m width and floodplains).

- Create a “Mask” folder to store the results created in this section

- Polygon to Raster to transform the channel polygon into a raster (match Output Coordinates, Processing Extent and Snap to computed flow direction raster)

Input: Geometry.gdb\channelpoly

Output: mask\frompoly

- Polyline to Raster to transform the multi-channel lines into a raster (match Output Coordinates, Processing Extent and Snap to computed flow direction raster)

Input: Geometry.gdb\routes

Output: mask\fromroutes

- Raster Calculator to have 1 where the raster made from the channel polygon has a value, or where the D4 has a value, or where there are multi channels. This ensures that the streams are always comprised of at least one cell.

Con(IsNull("fromroutes"),Con(IsNull("frompoly"),Con(IsNull("d4fd") == 0,1),1),1)

Output: mask\mask_temp

- Raster to Polygon to transform the channel mask back into a polygon in order to be able to subsequently modify it according to channel width.

Input: mask\mask_temp

Output: mask\mask_poly.shp

Simplify polygons unchecked

- Display width, and change the symbology to “Classified”. Use 2 classes, with a break value at 15m.

- Edit the mask_poly.shp file to cut out reaches that will be modelled with the SubGrid technique (width < 15 = no mask). NB: The mask should at best represent widths >15m. For reaches smaller than ~1km where the width varies, follow the general characteristic of the larger section, e.g. in the image below, the general characteristic is defined by green (=SubGrid), so this entire section should be cut out from the mask.

- Select the mask_poly.shp by clicking on it in an area where it overlaps with the D4fd. This should result in a selection that covers most of the mask_poly.shp. Only isolated cells and isolated secondary branches should not be included in the selection. Inverse this selection (so that now these isolated cells and isolated secondary branches get selected) and delete these attributes (see circled selection in image on the right).

    - Note: since Lisflood can only make connections between adjacent cells in 4 directions (so not diagonally), the presence of diagonal paths poses problems and need to be deleted.

- Polygon to Raster to transform the result back into a raster (match Output Coordinates, Processing Extent and Snap to computed flow direction raster).

Input: mask\mask_poly.shp (Value: gridcode, to ensure that the mask will have a value of 1 inside the channel)

Output: Lisflood_inputs\mask.tif

## Manning’s n for floodplain

This process shows how to generate Manning’s n for the floodplain, based on land cover data. The 2020 Land Cover of Canada is used hereafter, but this process can be modified for other land use data.

- Create a ManningFP.gdb to store the results create in this section

- Clip Raster the land cover raster previously downloaded over basin/study area

Output: ManningFP.gdb\LC_2020_clip

- Raster to Polygon to transform the landuse into a polygon in order to join it to the good nManning value.

Input: ManningFP.gdb\landuse_clip

Output: ManningFP.gdb\landuse_poly

Simplify polygons unchecked

- Add Join to created landcover shapefile with data from CanadaLC2015manning.csv, based on "covtype".

Input table: ManningFP.gdb\landuse_poly

Input join field : gridcode

Join table: CanadaLC2020manning.csv

Join table field : cov_type

- Polygon to Raster with Manning’s n assigned as Value Field (match Output Coordinates, Processing Extent and Snap to flow direction/DEM)

Input: ManningFP.gdb\landuse_poly

Output: Lisflood_inputs\n_floodplain.tif

# Simulations

Before running the simulations, there are certain steps to deal with first: adding boundary condition information to lakes and reservoirs, transferring the input variables into a folder, and cutting the network into tiles.

## Lakes and reservoirs

- Copy the polygons of the lakes

Input: Geometry.gdb\lakesfinal

Output: Lisflood_inputs\lakesforsim.shp

- Create an additional feature in lakes_forsim at the downstream end of the study area. Include at least one cell of the DEM. This is to assign the model a downstream boundary condition. The size of this polygon doesn’t matter, as long the river main line cross it.

- Optional: If lakes have extremely uneven shapes that, it is useful to create small polygons, roughly rectangle, where the river path flows into lakes. This will improve the creation of tiles.

- Optional: Additional features can be added to the lakes_forsim to exclude some areas from the hydraulic simulations.

- Add one field in lakes_forsim for each flood discharge to simulate. Store within the boundary condition (lake elevation) associated with each flood discharge for each lake.

## Tiling

The aim of this step is to divide the study network into multiple, smaller reaches, on which Lisflood is run independently, in order to ease processing. The tool to be used is Tiling.

Inputs:

10mDEMs.gdb\lidar10m_fd

Lisflood_inputs.gdb\lakesforsim

Geometry.gdb\from_pts

15000

3000

Tiles folder (NB: create a new empty folder called “Tiles” to store all outputs of this tool)

Temp folder

Two shapefiles are created by this tool, in the “Tiles” folder: polyzones.shp and sourcepoints.shp. The tiles are represented by the polygons in polyzones.shp. They must be checked and corrected if necessary. In particular, at confluences, a tile should not cut into meanders (see image below). Also, check in the attribute table of the polyzones that the “Lake_ID” corresponds to the lake that the tile (defined by the “grid_code”) is stopping at. Manually edit this if this is not the case.

Corner of a tile (used for the tributary in the upper-right corner) that need to be corrected, and the corrected tile.

The sourcepoints determine where the processing starts for that zone and should be located inside each polyzone. When editing the polyzones, ensure that the sourcepoint is inside it and located on the flow path defined by the flow accumulation raster.

If tiles are to be deleted, the corresponding source point should be deleted as well.

The simulation tiles will be based on the envelope of the shapes in polyzones.shp, which will be created in the next step. The modified polyzones do not have to be perfectly rectangular, but attention must be given to their shape in relation to how the envelope is going to be created:

Turquoise= modified polyzone; Green=envelope that will be created

## Hydraulic simulations with LISFLOOD-FP

The hydraulic simulations are run with two steps.

### Preparation of the simulations

- Hydraulic simulations preparation. This tool creates additional files in the “Tiles” folder as well as the input files to be used by Lisflood in the simulation folder (all variables clipped according to the zones in .txt format, .bci and .par files). NB: See Lisflood manual for more details about .bci, .par files.

Inputs:

10mDEMs.gdb\lidar10m_fd

10mDEMs.gdb\lidar10m_facc

Tiles

10mDEMs.gdb\lidar10m_avg

Lisflood_inputs\width_lisflood.tif

Lisflood_inputs\bathy_lisflood.tif

Lisflood_inputs\n_floodplain.tif

Lisflood_inputs\mask.tif

sims (NB: create before this new empty folder to store all outputs of this tool. This will also store the final results. Name can be changed).

Warning! If you model contains small tiles (sides of less than a few kilometers), you need to manually check the resulting bci files created by this tool. Make sure that the output window matches where water is expected to exit the domain (in small tiles it could be a too big window incorrectly encompassing the upstream side of the reach). See below an example:

A: small tile in between lakes, with direction of flow illustrated by turquoise arrow. The boundary condition is imposed at the exit of the flow path from the tile, encompassing a 4 km window (2 km on each side of the watercourse) to include the alluvial plain. In this example, since the distance from the flow exit point to the edge of the tile is 785 m (so less than 2 km), the boundary condition level is extended on the upstream end of the watercourse, on the northern edge of the tile (red marking along tile edges). B: To check if this happens, outbci points can be displayed in ArcGIS and the limits of the boundary condition checked. In this example, the boundary condition level for zone 10 is applied on 2 sides of the tile, E(ast) and N(orth) within the given coordinates. The side2 N in this example is wrongly assigned and should be deleted. C: To do this, find the .bci file in the sims folder, open with Notepad and edit accordingly. Here, the entire last line (selected) should be deleted.

### Flood discharges

Flood discharges (e.g. 20 year or 100 year floods) should be available at points, and can be estimated elsewhere with a similar process than the LiDAR discharges using the same equation:   
Q= QstAAstβ

with β obtained from a statistical study on gauging stations.

As for the LiDAR discharges, the input flood discharges (from the gauging stations) must be stored in a csv file, with the different discharges to simulate as rows and gauging stations as columns. A shapefile of the gauging stations must also be available (with their names as a field).

Example of discharges file:

| Discharge | s01BO001 | s01BO002 | s01BO003 |
| --- | --- | --- | --- |
| Q20 | 500 | 8 | 58 |
| Q100 | 600 | 10 | 65 |
| Q350 | 650 | 15 | 80 |

The process aims to add discharges values in the inbci.shp file in the “Tiles” folder.

- Make sure all points in inbci.shp falls on the D4 path of the rivers. Some mismatches occasionally occur due to the transformation of D8 flow into D4. This can be done by doing a Extract Multi Value to Points to add the data from Lisflood_inputs.gdb\d4fd to the inbci.shp file. Points in inbci.shp that are not on the D4 path should be manually move onto it.

- Locate features Along Routes should be used to locate the gauging stations on the D8 river network (if the flood discharge gauging stations differ from the ones used for assessing the LiDAR discharges)

Inputs: Shapefile of the gauging stations

Geometry.gdb\routesD8

RID

Adjust Search Radius depending on the situation

Output: Discharge.gdb\QStations_floods_D8

- Spatialize discharges from gauging stations – Q flood

Inputs: 10mDEMs.gdb\lidar10m_facc

Geometry.gdb\routesD8

RID

Geometry.gdb\linksD8

Geometry.gdb\pathpointsD8

Discharge.gdb\QStations_floods_D8

OBJECTID

Field with the gauging station name

Field with the drainage area of the stations

RID

MEAS

Field with the discharge at the stations. Re-run this tool several times and create multiple output files if several flood discharges are desired.

Beta coefficient

Output: Discharge.gdb\Qflood_D8

- Make Route Event Layer to visualize the points

Inputs: Geometry.gdb\routesD8

RID

Discharge.gdb\Qflood_D8

RID

POINT

dist

Output: Qflood_D8 Events

- Add Spatial Join to add the computed discharge to Tiles\inbci.shp (repeat with the different output of the previous step if several flood discharges are desired).

Input:

Tiles\inbci.shp

Discharge.gdb\ Qflood_D8 Events

Keep All Target Features checked

Match Option Closest

Permanently Join Fields checked

In the Fields section, remove all fields but the one with the discharge

### Running the simulations

- Run hydraulic simulations with LISFLOOD-FP. If flood simulations for multiple discharges are required, it is recommended to do the flood simulations for one discharge first, then check the result and correct possible errors before proceeding with the simulations for all discharges.

Inputs:

Simulations\zones

List of fields, in the inbci.shp shapefile, with the flood discharges to simulate

Simulations\sims (same folder used as output for Prepa, step 6.4.1)

filepath containing Lisflood executable on your computer

Lisflood_inputs\lakesforsim.shp

Fields in the lakesforsim.shp shapefile, with the downstream boundary conditions

Channel Manning’s n: 0.03 by default

Maximum simulation time: 300000 by default

Courant–Friedrichs–Lewy condition: 0.5 by default

Lisflood_inputs\bathy_lisflood.tif

log.txt (file created by the tool. Records warning and error messages of the simulation, for reference)

This tool creates separate folders for each specific discharge based on the name given in the fields in the inbci.shp shapefile.  Elev_zone# results, .bdy and .mass files can be found within these folders. The watershed-wide, merged result is res_XXX (with XXX the discharge field name) inside the sim_qprev folder.

Notes:

- For each discharge, a folder is created. Inside these folders, the simulation results for each tile is stored. The creation of these files can be checked to monitor the overall simulation progress during the simulations. If a warning “Steady state not reached” appears, the simulation results should be checked for the given discharge and tile. The .mass filescan be checked to look at the evolution of discharges input Qin and output Qout during the simulation. The Qin increases slowly until it reaches its nominal value, for stability purposes. The Qout should increase and reach the Qin (or almost) and the end of the simulation. Simulations should normally end automatically when the Qout is stable and close to the Qin. If the “Steady state not reached” warning appears, this simulation stopped instead when the maximum simulation time (in the tool parameter) was reached.

  - If the Qout is close to the Qin, the simulation is correct, the warning can be ignored.

  - If the Qout and/or the Qin behave chaotically (sudden increase/decrease, abnormal values), the simulation is unstable. Lowering the CFL criteria could help. If it doesn’t, changing the tile size (especially the downstream end) could help too.

  - If the Qout increase steadily but the simulation ends before it’s close to the Qin, the maximum simulation time can be increased. It is also recommended to look at the flood results to see if the water doesn’t reach some area where a steady-flow hydraulic model is not appropriate (water reaching another watershed, in particular).

- res_qXXX represents the merged end result of the simulation and contains flood elevation values in metres. It is created by Mosaic to New Raster of all tiles results, by using the maximum as a merging rule.

## Post-processing of simulation results

Post-processing can help create more accurate flood maps from the LISFLOOD-FP results. Post-processing can be done to improve/change 4 aspects of the results:

- Resolution can be improved;

- Elevations can be turned into flow depths;

- Hydraulically unconnected basins can be flooded;

- Incoherent data can be removed.

In addition, rasters can be transformed into polygons to create flood map.

A

B

Data post-processing, with depth at a finer resolution obtained, and unconnected basins flooded

As the post-processing can be tailored to the results a user want, only the main concepts are described here, not a step-by-step process.

Flood results must be propagated upstream using the tool Watershed. As this tool only accepts rasters with integer data, the flood results should be turned beforehand into a Integer (with Raster Calculator: Int(“raster”*100) so precision up to the centimetre is kept), and the result of the Watershed tool can be turned back to float after (with Raster Calculator: Float(“raster”)/100.). The result can be than compared to a fine resolution DEM (like the 1m LiDAR DEM). Such a process improves the resolution of the simulated floods and flood the disconnected basins. If an area is not considered floodable (an area lower in elevation than the flood but protected by levees), this should be identified and corrected manually.

Floods results (post-processed at a finer resolution or not) usually extend in not-simulated tributaries as well as upstream the extremities of the model. Results in these areas should be removed manually.

For map production, it is advised to remove data in the channels if flow depths are mapped, as bed elevation in the channels are incorrect. If polygons of flood zones are created, best results are obtained by reclassified raster results (0 if not flooded, 1 if flooded) and apply a Focal Statistics tool with “Majority” statistic to smooth the results (several consecutive run of the tool could be needed). Polygons can also be smoothed (Smooth polygons tool), and small islands/holes should be removed.